Safety Data Sheet

Issue Date: 01-Jan-2010	Revision Date: 20-Feb-2015	Version  1

|  | 1. IDENTIFICATION |  |
| --- | --- | --- |
| Product Identifier |  |  |
| Product Name | Spray-On, Fiberglass Cleaner , ½ Gallon and 1-Gallon |  |
| Other means of identification |  |  |
| SDS # | MLM-002 |  |
| Product Code | F1000, F1000C, F1004, F1004C |  |
| UN/ID No | UN3264 |  |
| Recommended use of the chemical and restrictions on use |  |  |
| Recommended Use | Aluminum Cleaner. |  |
| Details of the supplier of the safety data sheet |  |  |
| Manufacturer Address M&L Marine, Inc. PO Box 301 Hillview, KY 40129 |  |  |
| Emergency Telephone Number |  |  |
| Company Phone Number | (502) 969-7098 |  |
| Emergency Telephone (24 hr) | INFOTRAC 1-352-323-3500 (International) 1-800-535-5053 (North America) |  |
|  | 2. HAZARDS IDENTIFICATION |  |
| Appearance Clear, colorless liquid | Physical State Liquid | Odor Slight Acrid Odor |
| Classification |  |  |
| Acute toxicity - Oral |  | Category 4 |
| Acute toxicity - Dermal |  | Category 4 |
| Acute toxicity - Inhalation (Dusts/Mists) |  | Category 4 |
| Skin corrosion/irritation |  | Category 1 Sub-category C |
| Serious eye damage/eye irritation |  | Category 1 |
| Signal Word Danger |  |  |
| Hazard Statements Harmful if swallowed Harmful in contact with skin Harmful if inhaled Causes severe skin burns and eye damage |  |  |

| Precautionary Statements - Prevention Wash face, hands and any exposed skin thoroughly after handling Do not eat, drink or smoke when using this product Wear protective gloves/protective clothing/eye protection/face protection Use only outdoors or in a well-ventilated area Do not breathe dust/fume/gas/mist/vapors/spray |  |
| --- | --- |
| Precautionary Statements - Response |  |
| IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing Immediately call a poison center or doctor/physician |  |
| IF ON SKIN (or hair): Remove/Take off immediately all contaminated clothing. Rinse skin with water/shower Wash contaminated clothing before reuse Call a poison center or doctor/physician if you feel unwell |  |
| IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing Immediately call a poison center or doctor/physician |  |
| IF SWALLOWED: Rinse mouth. Do NOT induce vomiting Immediately call a poison center or doctor/physician |  |
| Precautionary Statements - Storage Store locked up |  |
| Precautionary Statements - Disposal Dispose of contents/container to an approved waste disposal plant |  |
| 3. COMPOSITION/INFORMATION ON INGREDIENTS Chemical Name CAS No Weight-% Sulfuric Acid 7664-93-9 <5 Hydrofluoric acid 7664-39-3 <1 **If Chemical Name/CAS No is "proprietary" and/or Weight-% is listed as a range, the specific chemical identity and/or percentage of composition has been withheld as a trade secret.** |  |
|  | 4. FIRST-AID MEASURES |
| First Aid Measures |  |
| Eye Contact | Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. Immediately call a poison center or doctor/physician. |
| Skin Contact | Remove/Take off immediately all contaminated clothing. Rinse skin with water/shower. Wash contaminated clothing before reuse. Call a poison center or doctor/physician if you feel unwell. |
| Inhalation | Remove victim to fresh air and keep at rest in a position comfortable for breathing. Immediately call a poison center or doctor/physician. |
| Ingestion | Rinse mouth. Do not induce vomiting. Give victim large quantities of water or milk. Never give anything by mouth to an unconscious person. Immediately call a poison center or doctor/physician. |
| Most important symptoms and effects |  |
| Symptoms | Causes severe skin burns and eye damage. Harmful if in contact with skin, swallowed, or inhaled. |
| Indication of any immediate medical attention and special treatment needed |  |
| Notes to Physician | Treat symptomatically. This product contains hydrofluoric acid. |

|  | 5. FIRE-FIGHTING MEASURES |
| --- | --- |
| Suitable Extinguishing Media Use extinguishing measures that are appropriate to local circumstances and the surrounding environment. |  |
| Unsuitable Extinguishing Media | Not determined. |
| Specific Hazards Arising from the Chemical Product is not flammable. |  |
| Protective equipment and precautions for firefighters As in any fire, wear self-contained breathing apparatus pressure-demand, MSHA/NIOSH (approved or equivalent) and full protective gear. |  |
|  | 6. ACCIDENTAL RELEASE MEASURES |
| Personal precautions, protective equipment and emergency procedures |  |
| Personal Precautions | Use personal protection recommended in Section 8. |
| Environmental Precautions | Prevent from entering into soil, ditches, sewers, waterways and/or groundwater. See Section 12, Ecological Information. See Section 13: DISPOSAL CONSIDERATIONS. |
| Methods and material for containment and cleaning up |  |
| Methods for Containment | Prevent further leakage or spillage if safe to do so. |
| Methods for Clean-Up | Neutralize the spilled product with soda ash. Contain and collect with an inert absorbent and place into an appropriate container for disposal. |
|  | 7. HANDLING AND STORAGE |
| Precautions for safe handling |  |
| Advice on Safe Handling | Handle in accordance with good industrial hygiene and safety practice. Use personal protection recommended in Section 8. Avoid contact with skin, eyes or clothing. Do not eat, drink or smoke when using this product. Wash face, hands, and any exposed skin thoroughly after handling. Do not breathe dust/fume/gas/mist/vapors/spray. Use only outdoors or in a well-ventilated area. Follow all product label instructions. Use only as directed. |
| Conditions for safe storage, including any incompatibilities |  |
| Storage Conditions | Keep container tightly closed and store in a cool, dry and well-ventilated place. Store locked up. |
| Incompatible Materials | Do not mix with other chemicals. |

8. EXPOSURE CONTROLS/PERSONAL PROTECTION

# Exposure Guidelines

| Chemical Name | ACGIH TLV | OSHA PEL | NIOSH IDLH |
| --- | --- | --- | --- |
| Sulfuric Acid 7664-93-9 | TWA: 0.2 mg/m3 thoracic fraction | TWA: 1 mg/m3 (vacated) TWA: 1 mg/m3 | IDLH: 15 mg/m3 TWA: 1 mg/m3 |
| Hydrochloric acid 7647-01-0 | Ceiling: 2 ppm | (vacated) Ceiling: 5 ppm (vacated) Ceiling: 7 mg/m3 Ceiling: 5 ppm Ceiling: 7 mg/m3 | IDLH: 50 ppm Ceiling: 5 ppm Ceiling: 7 mg/m3 |
| Phosphoric Acid 7664-38-2 | STEL: 3 mg/m3 TWA: 1 mg/m3 | TWA: 1 mg/m3 (vacated) TWA: 1 mg/m3 (vacated) STEL: 3 mg/m3 | IDLH: 1000 mg/m3 TWA: 1 mg/m3 STEL: 3 mg/m3 |
| Hydrofluoric acid 7664-39-3 | TWA: 0.5 ppm F TWA: 2.5 mg/m3 F S* Ceiling: 2 ppm F | TWA: 3 ppm F TWA: 2.5 mg/m3 F TWA: 2.5 mg/m3 dust (vacated) TWA: 3 ppm F (vacated) TWA: 2.5 mg/m3 (vacated) STEL: 6 ppm F | IDLH: 30 ppm Ceiling: 6 ppm 15 min Ceiling: 5 mg/m3 15 min TWA: 3 ppm TWA: 2.5 mg/m3 |

Appropriate engineering controls

Engineering Controls	Ensure adequate ventilation, especially in confined areas. Eyewash stations. Showers.

# Individual protection measures, such as personal protective equipment

Eye/Face Protection	Safety goggles or glasses.

Skin and Body Protection	Rubber gloves.

Respiratory Protection	Use in well-ventilated area.

General Hygiene Considerations Handle in accordance with good industrial hygiene and safety practice.

9. PHYSICAL AND CHEMICAL PROPERTIES

# Information on basic physical and chemical properties

| Physical State Appearance | Liquid Clear, colorless liquid | Odor | Slight Acrid Odor |
| --- | --- | --- | --- |
| Color | Clear, colorless | Odor Threshold | Not determined |
| Property | Values | Remarks • Method |  |
| pH Melting Point/Freezing Point Boiling Point/Boiling Range Flash Point Evaporation Rate Flammability (Solid, Gas) Upper Flammability Limits | Not applicable Not applicable Not applicable None >1 Liquid-Not applicable Not applicable | (butyl acetate = 1) |  |
| Lower Flammability Limit | Not applicable |  |  |
| Vapor Pressure Vapor Density Specific Gravity Water Solubility Solubility in other solvents Partition Coefficient Auto-ignition Temperature Decomposition Temperature Kinematic Viscosity Dynamic Viscosity Explosive Properties Oxidizing Properties | Not applicable Not applicable 1.17 + 0.1 Completely soluble Not determined Not determined Not determined Not determined Not determined Not determined Not determined Not determined | (Water = 1) |  |

10. STABILITY AND REACTIVITY

Reactivity

Not reactive under normal conditions.

# Chemical Stability

Stable under recommended storage conditions.

# Possibility of Hazardous Reactions

None under normal processing.

Hazardous Polymerization	Hazardous polymerization does not occur.

# Conditions to Avoid

Keep separated from incompatible substances. Keep out of reach of children.

# Incompatible Materials

Do not mix with other chemicals.

# Hazardous Decomposition Products

None known based on information supplied.

11. TOXICOLOGICAL INFORMATION

# Information on likely routes of exposure

Product Information

Eye Contact	Causes severe eye damage.

Skin Contact	Harmful in contact with skin. Causes severe skin burns.

Inhalation	Harmful if inhaled.

Ingestion	Harmful if swallowed.

# Component Information

| Chemical Name | Oral LD50 | Dermal LD50 | Inhalation LC50 |
| --- | --- | --- | --- |
| Sulfuric Acid 7664-93-9 | = 2140 mg/kg ( Rat ) | - | = 510 mg/m3 ( Rat ) 2 h |
| Hydrochloric acid 7647-01-0 | = 700 mg/kg ( Rat ) | > 5010 mg/kg ( Rabbit ) | = 3124 ppm ( Rat ) 1 h |
| Phosphoric Acid 7664-38-2 | = 1530 mg/kg ( Rat ) | = 2730 mg/kg ( Rabbit ) | > 850 mg/m3 ( Rat ) 1 h |
| Hydrofluoric acid 7664-39-3 | - | - | = 850 mg/m3 ( Rat ) 1 h = 1276 ppm ( Rat ) 1 h |

Information on physical, chemical and toxicological effects

Symptoms	Please see section 4 of this SDS for symptoms.

# Delayed and immediate effects as well as chronic effects from short and long-term exposure

Carcinogenicity	IARC has classified "strong inorganic acid mist containing sulfuric acid" as a Category 1 carcinogen, substance that is carcinogenic to humans. This classification does not apply to liquid forms of sulfuric acid. Inorganic mist is not generated under normal use of this product.

| Chemical Name | ACGIH | IARC | NTP | OSHA |
| --- | --- | --- | --- | --- |
| Sulfuric Acid 7664-93-9 | A2 | Group 1 | Known | X |
| Hydrochloric acid 7647-01-0 |  | Group 3 |  |  |

Legend

ACGIH (American Conference of Governmental Industrial Hygienists)

A2 - Suspected Human Carcinogen

IARC (International Agency for Research on Cancer)

Group 1 - Carcinogenic to Humans

Group 3 IARC components are "not classifiable as human carcinogens"

NTP (National Toxicology Program)

Known - Known Carcinogen

OSHA (Occupational Safety and Health Administration of the US Department of Labor)

X - Present

# Numerical measures of toxicity

Not determined

12. ECOLOGICAL INFORMATION

# Ecotoxicity

The product is not classified as environmentally hazardous. However, this does not exclude the possibility that large or frequent spills can have a harmful or damaging effect on the environment.

# Component Information

| Chemical Name | Algae/aquatic plants | Fish | Toxicity to microorganisms | Crustacea |
| --- | --- | --- | --- | --- |
| Sulfuric Acid 7664-93-9 |  | 500: 96 h Brachydanio rerio mg/L LC50 static |  | 29: 24 h Daphnia magna mg/L EC50 |
| Hydrochloric acid 7647-01-0 |  | 282: 96 h Gambusia affinis mg/L LC50 static |  |  |
| Hydrofluoric acid 7664-39-3 |  | 660: 48 h Leuciscus idus mg/L LC50 |  | 270: 48 h Daphnia species mg/L EC50 |

Persistence/Degradability

Not determined.

# Bioaccumulation

Not determined.

# Mobility

Chemical Name

Partition Coefficient

Hydrofluoric acid 7664-39-3

-1.4

Other Adverse Effects

Not determined

13. DISPOSAL CONSIDERATIONS

# Waste Treatment Methods

Disposal of Wastes	Disposal should be in accordance with applicable regional, national and local laws and regulations.

Contaminated Packaging	Disposal should be in accordance with applicable regional, national and local laws and regulations.

# US EPA Waste Number

| Chemical Name | RCRA | RCRA - Basis for Listing | RCRA - D Series Wastes | RCRA - U Series Wastes |
| --- | --- | --- | --- | --- |
| Hydrofluoric acid 7664-39-3 | U134 |  |  | U134 |

California Hazardous Waste Status

Chemical Name

California Hazardous Waste Status

Sulfuric Acid 7664-93-9

Toxic Corrosive

Phosphoric Acid 7664-38-2

Corrosive

14. TRANSPORT INFORMATION

Note	When shipped in containers of 1.3 gallons (5 L) or less this material may be reclassified in accordance with DOT regulation 49 CFR 173.154 as: Limited Quantity.

# DOT

UN/ID No	UN3264

Proper Shipping Name	Corrosive liquid, acidic, inorganic, n.o.s. (Sulfuric acid, Hydrochloric acid)

# Hazard Class	8

Packing Group	III

# IATA

UN/ID No	UN3264

Proper Shipping Name	Corrosive liquid, acidic, inorganic, n.o.s. (Sulfuric acid, Hydrochloric acid)

# Hazard Class	8

Packing Group	III

# IMDG

UN/ID No	UN3264

Proper Shipping Name	Corrosive liquid, acidic, inorganic, n.o.s. (Sulfuric acid, Hydrochloric acid)

# Hazard Class	8

Packing Group	III

15. REGULATORY INFORMATION

# International Inventories

| Chemical Name | TSCA | DSL | NDSL | EINECS | ELINCS | ENCS | IECSC | KECL | PICCS | AICS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sulfuric Acid | Present | X |  | Present |  | Present | X | Present | X | X |
| Hydrofluoric acid | Present | X |  | Present |  | Present | X | Present | X | X |

Legend:

TSCA - United States Toxic Substances Control Act Section 8(b) Inventory

DSL/NDSL - Canadian Domestic Substances List/Non-Domestic Substances List

EINECS/ELINCS - European Inventory of Existing Chemical Substances/European List of Notified Chemical Substances

ENCS - Japan Existing and New Chemical Substances IECSC - China Inventory of Existing Chemical Substances KECL - Korean Existing and Evaluated Chemical Substances

PICCS - Philippines Inventory of Chemicals and Chemical Substances

AICS - Australian Inventory of Chemical Substances

# US Federal Regulations

CERCLA

| Chemical Name | Hazardous Substances RQs | CERCLA/SARA RQ | Reportable Quantity (RQ) |
| --- | --- | --- | --- |
| Sulfuric Acid 7664-93-9 | 1000 lb | 1000 lb | RQ 1000 lb final RQ RQ 454 kg final RQ |
| Hydrochloric acid 7647-01-0 | 5000 lb | 5000 lb | RQ 5000 lb final RQ RQ 2270 kg final RQ |
| Phosphoric Acid 7664-38-2 | 5000 lb |  | RQ 5000 lb final RQ RQ 2270 kg final RQ |
| Hydrofluoric acid 7664-39-3 | 100 lb | 100 lb | RQ 100 lb final RQ RQ 45.4 kg final RQ |

SARA 313

| Chemical Name | CAS No | Weight-% | SARA 313 - Threshold Values % |
| --- | --- | --- | --- |
| Sulfuric Acid - 7664-93-9 | 7664-93-9 | <5 | 1.0 |
| Hydrochloric acid - 7647-01-0 | 7647-01-0 | <1 | 1.0 |
| Hydrofluoric acid - 7664-39-3 | 7664-39-3 | <1 | 1.0 |

CWA (Clean Water Act)

| Chemical Name | CWA - Reportable Quantities | CWA - Toxic Pollutants | CWA - Priority Pollutants | CWA - Hazardous Substances |
| --- | --- | --- | --- | --- |
| Sulfuric Acid | 1000 lb |  |  | X |
| Hydrochloric acid | 5000 lb |  |  | X |
| Phosphoric Acid | 5000 lb |  |  | X |
| Hydrofluoric acid | 100 lb |  |  | X |

US State Regulations

California Proposition 65

This product does not contain any Proposition 65 chemicals.

# U.S. State Right-to-Know Regulations

| Chemical Name | New Jersey | Massachusetts | Pennsylvania |
| --- | --- | --- | --- |
| Sulfuric Acid 7664-93-9 | X | X | X |
| Hydrochloric acid 7647-01-0 | X | X | X |
| Phosphoric Acid 7664-38-2 | X | X | X |
| Hydrofluoric acid 7664-39-3 | X | X | X |

| 16. OTHER INFORMATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| NFPA | Health Hazards Not determined | Flammability Not determined | Instability Not determined | Special Hazards Not determined |
| HMIS | Health Hazards 2 | Flammability 0 | Physical Hazards 1 | Personal Protection Not determined |
| Issue Date: | 01-Jan-2010 |  |  |  |
| Revision Date: | 20-Feb-2015 |  |  |  |
| Revision Note: | New format |  |  |  |
| Disclaimer The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at the date of its publication. The information given is designed only as a guidance for safe handling, use, processing, storage, transportation, disposal and release and is not to be considered a warranty or quality specification. The information relates only to the specific material designated and may not be valid for such material used in combination with any other materials or in any process, unless specified in the text. |  |  |  |  |
| End of Safety Data Sheet |  |  |  |  |